PREDICTIVE FAULT DETECTION IN SEMICONDUCTOR MANUFACTURING USING TIME-SERIES DEEP LEARNING REPORT

- Priyanka Prem Kumar

Task 1: Formulate a Predictive Problem

- Novel AI Challenge:

Semiconductor industry involves highly complex and sensitive equipment operating under strict process conditions (McKinsey & Company, 2021). Even minor deviations in sensor readings can lead to equipment faults, reduced yield, and significant financial losses. Traditional rule-based monitoring systems are often insufficient because they fail to capture nonlinear and temporal dependencies in high-dimensional sensor data (Sculley et al., 2015).

This project addresses the challenge of predictive fault detection using time series sensor data, where the goal itself is not to detect faults after they occur, but to anticipate failures in advance Lee et al. (2016). While fault detection is a well-studied problem, this work introduces a structured approach consisting of three phases:

- Phase 1: Baseline Modeling (static learning):

Develop traditional machine learning models (e.g., Logistic Regression, Random Forest) that treat each observation independently without temporal context. This establishes a performance benchmark.

- Phase 2: Temporal Modeling (sequence learning)

Incorporating temporal dependencies by modeling multivariate sensor data as sequences using deep learning architecture such as LSTM(Long Short-Term Memory) (Hochreiter & Schmidhuber, 1997). This phase captures evolving patterns leading to failures.

- Phase 3: Advanced Modeling (Early warning system)

Extending the temporal model to predict failures, several time steps ahead (early fault detection) and optionally also capture Transformer-based architecture to capture long-range dependencies (Vaswani et al., 2017).

This phased approach enables a comparative analysis between static and temporal models, demonstrating the value of sequence modeling in predictive maintenance. It reflects real world semiconductor manufacturing needs, where early intervention can significantly reduce downtime and improve product efficiency.

- Problem Statement:

- Objective: The objective is to develop a  predictive maintenance system that can forecast equipment faults in semiconductor manufacturing systems using historical sensor data. The system is designed to improve progressively from baseline models to advanced temporal models, ultimately enabling early fault detection.

- Input Data: The input consists of Multivariate sensor readings collected from semiconductor reading equipment. Each data point includes measurements from hundreds of sensors (e.g., temperature, pressure, voltage, etc.,.).The presentation of input data across phases:

Phase 1:  In Baseline model, each sample is treated as an independent feature vector.

X = xt

Phase 2 and 3: In Temporal Models, data is structured as time series sequences

X = [xt-n, xt-n+1, ......,xt-1 ]

- Expected Output:

Phase 1 and 2: Binary Classification output: 0: Normal Operation, 1: Imminent failure

Phase 3: To predict whether the failure will occur within next k time steps enabling early warning system

- Dataset description and Justification

This project utilizes SECOM manufacturing dataset, which is widely used  for studying fault detection in semiconductor processes.

- Why this Dataset?

- It contains real-world sensor data from semiconductor manufacturing processes.

- Includes high-dimensional features (hundreds of  sensor measurements), reflecting real world complexity.

- Provides labeled outcomes which indicate whether a process is a failure.

- Supports both static and temporal modeling after restructuring.

These factors make it  well-suited for multi-phase modeling approach, enabling comparison between baseline and  advanced methods.

- Dataset characteristics

This dataset has large number of sensor features(high dimensionality), presence of missing values across multiple sensors. Strong class imbalances as well as failures are rare compared to normal operations (He & Garcia, 2009).

- Potential challenges and Mitigation strategies

  - Missing data: Many sensor readings contain missing values due to equipment or logging issues.

Solution for phase 1: Mean/median imputation

Solution for phase 2 and 3: Forward filling or interpolation for temporal consistency

Feature removal if missingness is extensive

  - Class Imbalance: Failure events are significantly less frequent than normal operations.

Solution for phase 1: Oversampling techniques such as SMOTE.

Solution for phase 2 and 3: Class-Weighted loss functions.

Use of evaluation metrics such as F1-score and ROC-AUC (Davis & Goadrich , 2006).

  - High Dimensionality:

The dataset has  a large number of sensor features, which may introduce noise and redundancy.

Solution for phase 1: feature selection using dimensionality reduction(e.g.: PCA) or use regularization techniques.

Solution for phase 2 and 3: learning feature importance over time

All in all, the SECOM dataset provides a realistic and challenging environment for modeling semiconductor faults. Its complexity, characterized by missing data, imbalance, and high dimensionality, makes it ideal for demonstrating a progressive AI pipeline i.e. Establish baseline performance using static models, improve performance using temporal deep learning (LSTM) and enable early fault prediction and explore advanced architectures (Transformer). This progression not only reflects real-world industrial systems but also highlights the advantages of temporal modeling for predictive maintenance in semiconductor manufacturing.

TASK 2: Designing and Implementing a Methodology

This study adopts a progressive modeling framework to address failure prediction in the SECOM semiconductor manufacturing dataset. The methodology is designed to handle key challenges, including high dimensionality, missing data, temporal dependencies, and severe class imbalance, through three modeling phases: Logistic Regression (baseline), LSTM (temporal modeling), and Transformer (early warning system).

Phase 1: Baseline Model - Logistic Regression

Methodology Design: The first phase establishes a baseline using a Logistic Regression model, providing a simple framework for binary classification. The goal is to evaluate whether meaningful predictive signals exist in the dataset before introducing complex models.

- The data preprocessing pipeline began with the removal of features containing more than 50% missing values to reduce noise and improve data quality. The remaining missing values were imputed using the mean computed from the training set, ensuring no data leakage. All features were then standardized using a StandardScaler to normalize the input space. The target labels were converted from -1, 1 to 0, 1 respectively for compatibility with binary classification. A stratified train-test split was performed to preserve the original class distribution.

- To address class imbalance, class weights were computed based on the distribution of the training data. These weights were incorporated into the loss function using BCEWithLogitsLoss with a pos_weight parameter, allowing the model to penalize misclassification of the minority class more heavily.

- For model implementation, a single-layer linear model (logistic regression) was built using PyTorch. The model was trained using the Adam optimizer over multiple epochs, with training loss monitored to ensure convergence.

- A validation strategy was incorporated to improve decision-making performance. A separate validation set was used to tune the classification threshold, and the optimal threshold was selected by maximizing the F1-score, ensuring better balance between precision and recall in the presence of class imbalance.

Expected Outcome: This phase is expected to provide a reference benchmark, demonstrating moderate ranking performance but limited ability to detect rare failure events due to the lack of temporal modeling.

Phase 2: Temporal Modeling - LSTM

Methodology Design: The second phase introduces a Long Short-Term Memory (LSTM) network to capture temporal dependencies in sequential sensor data. The objective is to model how sensor readings evolve over time leading to failure events.

- The data preparation process involved sorting the dataset by timestamp, followed by a time-based split into training, validation, and test sets without shuffling, ensuring that future information does not leak into past observations. The data was then transformed into fixed-length sequences of size SEQ_LEN, enabling the model to learn from temporal context.

- To address the weak temporal signals present in raw sensor data, additional temporal features were engineered. These included rolling means to capture underlying trends, rolling standard deviation to reflect variability and instability, and first-order differences (delta) to highlight sudden changes in sensor readings. Together, these features enhance the representation of pre-failure patterns and make them more explicit for the model.

- Sequence labeling was performed at the sequence level, where each sequence was assigned a label based on whether a failure occurs at the next timestep. This formulation allows the model to learn short-term temporal dependencies leading to failure events.

- To handle class imbalance, class-weighted BCEWithLogitsLoss was used, assigning higher importance to the minority class during training. Aggressive oversampling was intentionally avoided to prevent bias and overfitting.

- The model architecture consists of a multi-layer bidirectional LSTM, which captures temporal dependencies in both forward and backward directions. Dropout was incorporated for regularization, and a final fully connected layer was used to produce binary predictions.

- The training strategy employed mini-batch training using a DataLoader for efficient optimization. Early stopping was implemented based on validation ROC-AUC to prevent overfitting, and threshold tuning was performed on the validation set to select the optimal decision boundary by maximizing the F1-score.

Expected Outcome: The LSTM is expected to highlight challenges in learning stable temporal patterns from noisy sensor data and motivates more advanced approaches.

Phase 3: Advanced Modeling – Transformer-Based Early Warning System

Methodology Design: The final phase introduces a Transformer-based architecture combined with a reformulation of the prediction task into an early warning problem. Instead of predicting failures at the next timestep, the model predicts whether a failure will occur within a future horizon of Ksteps.

- The problem formulation was modified to enable early fault detection rather than predicting failures at a single timestep. Each input sequence was labeled as positive if a failure occurred within a future window of K timesteps, i.e., max⁡(yt+1,…,yt+K)=1. This reformulation allows the model to learn precursor patterns and provide proactive warnings instead of reactive predictions.

- The feature representation builds upon Phase 2 by retaining the same engineered temporal features, including rolling statistics and first-order differences. In addition, sequence-level representations were enhanced using mean pooling, enabling better aggregation of information across the entire sequence and improving the signal-to-noise ratio.

- Model architecture is based on a Transformer encoder with a self-attention mechanism, which allows the model to capture long-range dependencies more effectively than recurrent architecture. Unlike LSTMs, the Transformer processes sequence elements in parallel, improving both efficiency and its ability to model complex temporal interactions.

- To handle class imbalance, a class-weighted loss function was used, with careful tuning to avoid overfitting to the minority class. The training process followed a proper train-validation-test split, ensuring no temporal leakage. Threshold tuning was performed on the validation set to optimize classification performance, and the model was evaluated using F1-score, ROC-AUC, and Average Precision (PR-AUC).

Expected Outcome: The Transformer model is expected to demonstrate the model’s ability to capture complex temporal dependencies, effectively identify failure patterns, and provide early warnings for proactive maintenance.

TASK 3: Evaluating the Model

The performance of the models was evaluated by using multiple metrics to ensure a comprehensive assessment under severe class imbalance. Specifically, F1-score, ROC-AUC, and Average Precision (PR-AUC) were used, along with validation-based threshold tuning and analysis of training dynamics.

Evaluation Metrics: F1-score was used as the primary metric because it balances precision and recall, making it particularly suitable for imbalanced datasets where failure events are rare. ROC-AUC was used to evaluate the model’s ability to rank positive and negative samples across all thresholds. However, since ROC-AUC can be overly optimistic in imbalanced settings, Average Precision (PR-AUC) was also used to measure how effectively the model identifies the minority class.

Threshold Tuning: Instead of using a default classification threshold of 0.5, a validation-based threshold tuning strategy was applied. Predicted probabilities on the validation set were evaluated across a range of thresholds, and the threshold that maximized the F1-score was selected. This approach improves the balance between precision and recall and ensures better detection of rare failure events.

Training Behavior and Generalization: The training dynamics reveal important differences in model behavior.

- The Logistic Regression model showed stable convergence, with loss decreasing smoothly, indicating consistent optimization but limited representational capacity.

- In contrast, the LSTM model exhibited rapid reduction in training loss, while validation ROC-AUC fluctuated and peaked early before declining. This indicates overfitting and limited generalization, suggesting that the model learns patterns specific to the training data without capturing robust predictive structure.

- The Transformer model demonstrated more stable validation performance, with consistent improvements and better alignment between training and validation metrics. This indicates improved generalization and a stronger ability to learn meaningful temporal dependencies.

Interpretation of Metrics: The evaluation highlights key differences in model performance.

- Logistic Regression achieves moderate ROC-AUC, indicating some ability to distinguish between classes, but struggles with low F1 and PR-AUC due to class imbalance.

- The LSTM improves F1-score and PR-AUC, showing better detection of failure events at specific thresholds. However, its ROC-AUC decreases compared to the baseline, indicating weaker overall ranking performance and instability in learning probability distributions.

- The Transformer achieves strong performance across all metrics, with significant improvements in F1-score and PR-AUC. This indicates superior ability to identify rare failure events, while the higher ROC-AUC reflects improved ranking capability.

Overall Evaluation: Overall, the evaluation demonstrates that increasing model complexity alone is insufficient; proper handling of temporal structure, class imbalance, and threshold calibration is essential. The Transformer model, combined with improved problem formulation, provides the most reliable and generalizable performance.

TASK 4: Presenting Results and Discussion

- Summarizing Results

| PHASE | MODEL | F1 Score | ROC-AUC | PR-AUC |
| --- | --- | --- | --- | --- |
| 1 | Logistic Regression | 0.1500 | 0.6370 | 0.1191 |
| 2 | LSTM | 0.2095 | 0.5937 | 0.1899 |
| 3 | Transformer | 0.4590 | 0.7519 | 0.4752 |

Table 1: Evaluation performance metrics vs Phases

In Phase 1, a Logistic Regression model was implemented as a baseline using static sensor features without incorporating temporal dependencies. The model achieved moderate ROC-AUC, indicating some ability to distinguish between normal and failure cases, but performed poorly in terms of F1-score and Average Precision. This reflects its inability to handle severe class imbalance and capture evolving patterns in sensor data. As a result, it serves as a useful benchmark but is insufficient for predictive maintenance tasks.

In Phase 2, an LSTM-based model was introduced to incorporate temporal dependencies through sequential input data. This resulted in improved F1-score and Average Precision, indicating better detection of failure events compared to the baseline. However, the ROC-AUC decreased slightly, suggesting that the model struggled to maintain consistent ranking performance across all samples. This limitation highlights the challenges of learning from noisy, high-dimensional sensor data and the constraints of recurrent architectures in capturing complex temporal relationships.

In Phase 3, a Transformer-based model was implemented with an improved problem formulation for early fault detection. By leveraging self-attention and sequence-level representations, the model was able to capture long-range dependencies and complex temporal patterns. This resulted in substantial improvements across all metrics, particularly in F1-score and Average Precision, demonstrating significantly better detection of rare failure events. The higher ROC-AUC further confirms improved ranking capability. Importantly, the model provides early warnings of failures, making it more suitable for real-world predictive maintenance applications.

Figure 1: Model Performance Comparison across Phases

Figure 1 illustrates the performance trends across the three phases, showing clear improvement as the models evolve from static to temporal and finally to attention-based architectures. Logistic Regression provides a reasonable baseline, while LSTM improves failure detection by incorporating temporal information. However, the Transformer achieves the best overall performance, particularly in handling class imbalance and detecting rare events.

Conclusion: The results demonstrate that incorporating temporal modeling and reformulating the prediction problem are critical for effective predictive maintenance. While Logistic Regression establishes a baseline and LSTM introduces temporal learning, the Transformer model provides the most robust and scalable solution. Its ability to capture complex dependencies and generate early warnings makes it the most suitable approach for fault detection in semiconductor manufacturing systems.

- Successes, Limitations, and Areas for improvement

This project demonstrates a clear progression in predictive performance across three phases, Logistic Regression, LSTM, and Transformer, applied to the SECOM semiconductor manufacturing dataset. Each phase addresses key challenges such as high dimensionality, temporal dependencies, and severe class imbalance, progressively improving predictive capability for fault detection.

The Logistic Regression baseline confirms that predictive signal exists in the dataset, achieving moderate ROC-AUC. However, its low F1-score and Average Precision highlight its inability to capture temporal structure and effectively handle rare failure events. This reflects known limitations of linear models in imbalanced settings, where ROC-AUC alone can be misleading and Precision-Recall metrics are more informative (Jesse Davis & Goadrich, 2006). The LSTM model introduces temporal learning through sequential dependencies and improves F1-score and Average Precision compared to the baseline, indicating better detection of failure events. However, ROC-AUC does not improve and slightly declines, suggesting inconsistent ranking performance. This highlights the difficulty of learning robust temporal representations from noisy, high-dimensional sensor data and the limitations of recurrent architectures under class imbalance (Hochreiter & Schmidhuber, 1997; He & Garcia, 2009). The Transformer model addresses these limitations using self-attention and reformulates the task as an early warning problem by predicting failures within a future horizon of K steps. It achieves the best overall performance, with substantial gains in F1-score and Average Precision, demonstrating strong capability in detecting rare failure events and capturing long-range temporal dependencies (Vaswani et al., 2017).

Despite these gains, several improvements are still possible. First, model performance could benefit from more systematic hyperparameter optimization, particularly for sequence length, prediction horizon K, and Transformer depth. Second, advanced imbalance strategies such as focal loss or calibrated sampling could further improve robustness beyond simple class weighting. Third, incorporating domain-driven feature engineering or sensor selection could reduce noise in high-dimensional inputs. Finally, improving model interpretability (e.g., attention analysis or feature attribution methods) would make the system more actionable in industrial deployment settings.

The Transformer model is computationally more expensive and sensitive to hyperparameter choices. Additionally, imbalance handling techniques may introduce overfitting risks, and model complexity reduces interpretability compared to simpler models. Overall, the results highlight that combining temporal modeling with appropriate problem reformulation and advanced architecture is essential for effective predictive maintenance in high-stakes industrial environments.

- Real-world significance, Potential impact of your predictions, and Future work.

The predictive framework developed in this study has strong real-world relevance for semiconductor manufacturing, where minor process deviations can lead to significant yield loss, downtime, and financial impact. By moving from static classification to a temporal early warning system, the Transformer-based model enables proactive fault detection, allowing intervention before failures occur. This shift from reactive to predictive maintenance is widely recognized as essential in modern industrial systems and can significantly reduce operational costs and downtime (McKinsey & Company, 2021; Jay Lee et al., 2016). The improvements in F1-score and Average Precision further indicate that the model is well-suited for highly imbalanced settings, where false negatives are particularly costly (Haibo He & Garcia, 2009).

From an operational perspective, the system can be integrated into manufacturing pipelines to continuously monitor sensor streams and flag high-risk sequences. These predictions can trigger preventive actions such as recalibration, inspection, or controlled shutdowns, improve overall equipment effectiveness (OEE) and reducing scrap rates. The early warning formulation, predicting failures within a future horizon K, closely aligns with industrial requirements where lead time is critical for decision-making (Edward Chow et al., 2001).

Several directions can further improve the system. First, interpretability can be enhanced by analyzing Transformer attention patterns to identify critical sensors and time steps influencing predictions. Second, the prediction horizon K should be optimized based on operational constraints to balance early detection with reliability. Third, more advanced imbalance handling techniques such as focal loss or adaptive reweighting could further improve rare event detection (Tsung-Yi Lin et al., 2017).

Additionally, incorporating domain knowledge such as known fault modes or sensor groupings may improve feature quality and robustness. Real-world deployment considerations, including latency, streaming inference, and model drift, should also be addressed for production scalability, as performance degradation over time is a known challenge in machine learning systems (D. Sculley et al., 2015). Finally, extending the framework to multimodal data sources or applying transfer learning across manufacturing lines could further enhance generalization. In summary, this work demonstrates that combining temporal modeling with an early warning formulation significantly improves predictive maintenance systems, with strong potential for real-world industrial impact.

REFERENCE:

- Jesse Davis, J., & Goadrich, M. (2006). The relationship between Precision-Recall and ROC curves. Proceedings of the 23rd International Conference on Machine Learning (ICML).

- Haibo He, H., & Garcia, E. A. (2009). Learning from imbalanced data. IEEE Transactions on Knowledge and Data Engineering, 21(9), 1263–1284.

- Sepp Hochreiter, S., & Jürgen Schmidhuber, J. (1997). Long Short-Term Memory. Neural Computation, 9(8), 1735–1780.

- Ashish Vaswani, A., et al. (2017). Attention Is All You Need. Advances in Neural Information Processing Systems (NeurIPS).

- Jay Lee, J., Bagheri, B., & Jin, C. (2016). Introduction to cyber manufacturing. Manufacturing Letters, 8, 11–15.

- McKinsey & Company (2021). The value of predictive maintenance in manufacturing.

- D. Sculley, D., et al. (2015). Hidden technical debt in machine learning systems. Advances in Neural Information Processing Systems (NeurIPS).

- Edward Chow, E., et al. (2001). Time series forecasting for fault detection. IEEE Transactions on Industrial Electronics.

- Tsung-Yi Lin, T.-Y., et al. (2017). Focal Loss for Dense Object Detection. Proceedings of ICCV.

***********************************************